CCNA Security

Lab - Configure a Site-to-Site IPsec VPN Using ISR CLI and ASA 5506-X ASDM (Instructor Version)

Instructor Note: Red font color or gray highlights indicate text that appears in the instructor copy only.

Topology

Note: ISR G2 devices use GigabitEthernet interfaces instead of FastEthernet interfaces.

# IP Addressing Table

| Device | Interface | IP Address | Subnet Mask | Default Gateway | Switch Port |
| --- | --- | --- | --- | --- | --- |
| R1 | E0/3 | 209.165.200.225 | 255.255.255.248 | N/A | ASA G0/0 |
| R1 | S1/1 (DCE) | 10.1.1.1 | 255.255.255.252 | N/A | N/A |
| R2 | S1/1 | 10.1.1.2 | 255.255.255.252 | N/A | N/A |
| R2 | S1/0 (DCE) | 10.2.2.2 | 255.255.255.252 | N/A | N/A |
| R3 | E0/3 | 172.16.3.1 | 255.255.255.0 | N/A | S3 E0/0 |
| R3 | S1/0 | 10.2.2.1 | 255.255.255.252 | N/A | N/A |
| ASA | G0/0 (outside) | 209.165.200.226 | 255.255.255.248 | NA | R1 E0/3 |
| ASA | G0/1 (inside) | 192.168.1.1 | 255.255.255.0 | NA | S2 E0/0 |
| ASA | G0/2 (dmz) | 192.168.2.1 | 255.255.255.0 | NA | S1 E0/0 |
| PC-A | NIC | 192.168.2.3 | 255.255.255.0 | 192.168.2.1 | S1 E0/1 |
| PC-B | NIC | 192.168.1.3 | 255.255.255.0 | 192.168.1.1 | S2 E0/1 |
| PC-C | NIC | 172.16.3.3 | 255.255.255.0 | 172.16.3.1 | S3 E0/1 |

Objectives

Part 1: Configure Basic Device Settings

Cable the network and clear previous device settings, as shown in the topology.

Configure basic settings for routers.

Configure PC host IP settings.

Verify connectivity.

Save the basic running configuration for each router and switch.

Part 2: Access the ASA Console and ASDM

Access the ASA console.

Clear the previous ASA configuration settings.

Bypass Setup mode.

Use the CLI command script to configure the ASA.

Verify HTTP ASDM access.

Part 3: Configure the ISR as a Site-to-Site IPsec VPN Endpoint Using the CLI

Configure basic VPN connection information settings.

Specify IKE policy parameters.

Configure a transform set.

Specify traffic to protect.

Review the summary of the configuration.

Review the site-to-site VPN tunnel configuration.

Part 4: Configure the ASA as a Site-to-Site IPsec VPN Endpoint Using ASDM

Access ASDM.

Review the ASDM Home screen.

Start the VPN wizard.

Configure peer device identification.

Specify the traffic to protect.

Configure authentication.

Configure miscellaneous settings.

Review the configuration summary and deliver the commands to the ASA.

Verify the ASDM VPN connection profile.

Test the VPN configuration from R3.

Use ASDM monitoring to verify the tunnel.

Background/Scenario

In addition to acting as a remote access VPN concentrator, the ASA can provide site-to-site IPsec VPN tunneling. The tunnel can be configured between two ASAs or between an ASA and another IPsec VPN-capable device, such as an ISR, as is the case with this lab.

In this scenario, your company has two locations connected to an ISP. R1 represents a customer-premise equipment (CPE) device managed by the ISP. R2 represents an intermediate Internet router. R3 connects users at the remote branch office to the ISP. The ASA is an edge security device that connects the internal corporate network and DMZ to the ISP while providing NAT services to inside hosts.

Management has asked you to provide a dedicated site-to-site IPsec VPN tunnel between the ISR router at the remote branch office and the ASA device at the corporate site. This tunnel will protect traffic between the branch office LAN and the corporate LAN, as it passes through the Internet. The site-to-site VPN does not require a VPN client on the remote or corporate site host computers. Traffic from either LAN to other Internet destinations is routed by the ISP and is not protected by the VPN tunnel. The VPN tunnel will pass through R1 and R2; both routers are not aware of the tunnel’s existence.

In Part 1 of this lab, you will configure the topology and non-ASA devices. In Part 2, you will prepare the ASA for ASDM access. In Part 3, you will use the CLI to configure the R3 ISR as a site-to-site IPsec VPN endpoint. In Part 4, you will configure the ASA as a site-to-site IPsec VPN endpoint using the ASDM VPN wizard.

Note: The router commands and output in this lab are from a Cisco 1941 router with Cisco IOS Release 15.4(3)M2 (with a Security Technology Package license). Other routers and Cisco IOS versions can be used. See the Router Interface Summary Table at the end of this lab to determine which interface identifiers to use based on the equipment in the lab. Depending on the router model and Cisco IOS version, the commands available and the output produced might vary from what is shown in this lab.

The ASA used with this lab is a Cisco model 5506-X with an 8-port integrated switch, running OS version 9.10(1), Adaptive Security Device Manager (ASDM) version 7.10(1), and comes with a Base license that allows a maximum of five VLANs.

Note: Before beginning, ensure that the routers and switches have been erased and have no startup configurations.

Instructor Note: Instructions for erasing switches and routers are provided in Chapter 0.0.0.0. Instructions for erasing the ASA, accessing the console, and accessing ASDM are provided in this lab.

- Required Resources

3 Routers (Cisco 1941 with Cisco IOS Release 15.4(3)M2 image with a Security Technology Package license)

3 Switches (Cisco 2960 with cryptography IOS image for SSH support – Release 15.0(2)SE7 or comparable) (not required)

1 ASA 5506-X (OS version 9.10(1) and ASDM version 7.10(1) and Base license or comparable)

3 PCs (Windows, SSH Client and Java version compatible with installed ASDM version)

- Serial and Ethernet cables, as shown in the topology

- Console cables to configure Cisco networking devices

Instructor Notes:

This lab has four main parts. Parts 1 and 2 can be performed separately but must be performed before parts 3 and 4. Part 2 prepares the ASA for ASDM access. Part 3 configures the R3 ISR as a site-to-site IPsec VPN endpoint using the CLI. Part 4 configures the opposite end of the tunnel on the ASA using ASDM. Parts 3 and 4 should be performed sequentially. Each part will use CLI and ASDM as required to verify the configuration.

The main goal is to configure a site-to-site IPsec VPN between two sites using an ISR at one end of the tunnel and an ASA at the other end.

The final running configs for all devices are found at the end of the lab.

- Configure Basic Device Settings

In Part 1, you will set up the network topology and configure basic settings on the routers, such as IP addresses and static routing.

Note: Do not configure any ASA settings at this time.

Cable the network and clear previous device settings.

Attach the devices shown in the topology diagram and cable as necessary. Ensure that the routers and switches have been erased and have no startup configurations.

Configure R1 using the CLI script.

In this step, you will use the following CLI script to configure basic settings on R1. Copy and paste the basic configuration script commands listed below. Observe the messages as the commands are applied to ensure that there are no warnings or errors.

Note: Depending on the router model, interfaces might be numbered differently than those listed. You might need to alter the designations accordingly.

Note: Passwords in this task are set to a minimum of 10 characters and are relatively simple for the purposes of performing the lab. More complex passwords are recommended in a production network.

hostname R1

security passwords min-length 10

enable algorithm-type scrypt secret cisco12345

username admin01 algorithm-type scrypt secret admin01pass

ip domain name ccnasecurity.com

line con 0

login local

exec-timeout 5 0

logging synchronous

exit

line vty 0 4

login local

transport input ssh

exec-timeout 5 0

logging synchronous

exit

interface  e0/3

ip address 209.165.200.225 255.255.255.248

no shut

exit

interface Serial1/1

ip address 10.1.1.1 255.255.255.252

clock rate 2000000

no shut

exit

ip route 0.0.0.0 0.0.0.0 Serial1/1

crypto key generate rsa general-keys modulus 1024

Configure R2 using the CLI script.

In this step, you will use the following CLI script to configure basic settings on R2. Copy and paste the basic configuration script commands listed below. Observe the messages as the commands are applied to ensure that there are no warnings or errors.

hostname R2

security passwords min-length 10

enable algorithm-type scrypt secret cisco12345

username admin01 algorithm-type scrypt secret admin01pass

ip domain name ccnasecurity.com

line con 0

login local

exec-timeout 5 0

logging synchronous

exit

line vty 0 4

login local

transport input ssh

exec-timeout 5 0

logging synchronous

exit

interface Serial1/1

ip address 10.1.1.2 255.255.255.252

no shut

exit

interface Serial1/0

ip address 10.2.2.2 255.255.255.252

clock rate 2000000

no shut

exit

ip route 209.165.200.224 255.255.255.248 Serial1/1

ip route 172.16.3.0 255.255.255.0 Serial1/0

crypto key generate rsa general-keys modulus 1024

Configure R3 using the CLI script.

In this step, you will use the following CLI script to configure basic settings on R3. Copy and paste the basic configuration script commands listed below. Observe the messages as the commands are applied to ensure that there are no warnings or errors.

hostname R3

security passwords min-length 10

enable algorithm-type scrypt secret cisco12345

username admin01 algorithm-type scrypt secret admin01pass

ip domain name ccnasecurity.com

line con 0

login local

exec-timeout 5 0

logging synchronous

exit

line vty 0 4

login local

transport input ssh

exec-timeout 5 0

logging synchronous

exit

interface e0/3

ip address 172.16.3.1 255.255.255.0

no shut

exit

int Serial1/0

ip address 10.2.2.1 255.255.255.252

no shut

exit

ip route 0.0.0.0 0.0.0.0 Serial1/0

crypto key generate rsa general-keys modulus 1024

Configure PC host IP settings.

Configure a static IP address, subnet mask, and default gateway for PC-A, PC-B, and PC-C as shown in the IP Addressing table.

    - Verify connectivity.

Because the ASA is the focal point for the network zones, and it has not yet been configured, there will be no connectivity between devices that are connected to it. However, PC-C should be able to ping the R1 interface E0/3. From PC-C, ping the R1 E0/3 IP address (209.165.200.225). If these pings are unsuccessful, troubleshoot the basic device configurations before continuing.

Note: If you can ping from PC-C to R1 E0/3 and S1/1, you have demonstrated that static routing is configured and functioning correctly.

Save the running configuration for each router.

- Access the ASA Console and ASDM

Clear the previous ASA configuration settings.

Use the write erase command to remove the startup-config file from flash memory.

Note: The erase startup-config IOS command is not supported on the ASA.

Use the reload command to restart the ASA. This causes the ASA to display in CLI Setup mode. If you see the System config has been modified. Save? [Y]es/[N]o: message, type n, and press Enter.

Bypass Setup mode.

When the ASA completes the reload process, it should detect that the startup configuration file is missing and go into Setup mode. If it does go into Setup mode, repeat Step 2.

When prompted to preconfigure the firewall through interactive prompts (Setup mode), respond with no. If you have inadvertently started the setup wizard, press CTRL-Z to exit it. The terminal screen should display the default ASA user EXEC hostname and prompt ciscoasa>.

Enter privileged EXEC mode with the enable command. The password should be kept blank (no password).

Configure the ASA by using the CLI script.

In this step, you will use a CLI script to configure basic settings, the firewall, and the DMZ.

Use the show run command to confirm that there is no previous configuration in the ASA other than the defaults that the ASA automatically inserts.

Enter global configuration mode. When prompted to enable anonymous call-home reporting, respond no.

Copy and paste the Pre-VPN Configuration Script commands listed below at the ASA global configuration mode prompt to start configuring the SSL VPNs. Observe the messages as the commands are applied to ensure that there are no warnings or errors. If prompted to replace the RSA key pair, respond yes.

hostname CCNAS-ASA

domain-name ccnasecurity.com

enable password cisco12345

interface G0/1

nameif inside

security-level 100

ip address 192.168.1.1 255.255.255.0

no shutdown

interface G0/0

nameif outside

security-level 0

ip address 209.165.200.226 255.255.255.248

no shutdown

interface G0/2

nameif dmz

security-level 70

ip address 192.168.2.1 255.255.255.0

no shutdown

object network inside-net

subnet 192.168.1.0 255.255.255.0

object network dmz-server

host 192.168.2.3

access-list OUTSIDE-DMZ extended permit ip any host 192.168.2.3

object network inside-net

nat (inside,outside) dynamic interface

object network dmz-server

nat (dmz,outside) static 209.165.200.227

access-group OUTSIDE-DMZ in interface outside

route outside 0.0.0.0 0.0.0.0 209.165.200.225 1

username admin01 password admin01pass

aaa authentication ssh console LOCAL

aaa authentication http console LOCAL

http server enable

http 192.168.1.0 255.255.255.0 inside

ssh 192.168.1.0 255.255.255.0 inside

ssh timeout 10

class-map inspection_default

match default-inspection-traffic

policy-map global_policy

class inspection_default

inspect icmp

crypto key generate rsa modulus 1024

At the privileged EXEC mode prompt, issue the write mem (or copy run start) command to save the running configuration to the startup configuration and the RSA keys to non-volatile memory.

- Configure the ISR as a Site-to-Site IPsec VPN Endpoint Using the CLI

In Part 3 of this lab, you will configure R3 as an IPsec VPN endpoint for the tunnel between R3 and the ASA. R1 and R2 are unaware of the tunnel.

Verify connectivity from the R3 LAN to the ASA.

In this step, you will verify that PC-C on the R3 LAN can ping the ASA outside interface.

Ping the ASA IP address of 209.165.200.226 from PC-C.

PC-C:\> ping 209.165.200.226

If the pings are unsuccessful, troubleshoot the basic device configurations before continuing.

Enable IKE policies on R3.

IPsec is an open framework that allows for the exchange of security protocols as new technologies and encryption algorithms are developed.

There are two central configuration elements in the implementation of an IPsec VPN:

Implement Internet Key Exchange (IKE) parameters.

Implement IPsec parameters.

Verify that IKE is supported and enabled.

IKE Phase 1 defines the key exchange method used to pass and validate IKE policies between peers. In IKE Phase 2, the peers exchange and match IPsec policies for the authentication and encryption of data traffic.

IKE must be enabled for IPsec to function. IKE is enabled, by default, on IOS images with cryptographic feature sets. If it is disabled, you can enable it with the crypto isakmp enable command. Use this command to verify that the router IOS supports IKE and that it is enabled.

R3(config)# crypto isakmp enable

Note: If you cannot execute this command on the router, you must upgrade to the IOS image that includes the Cisco cryptographic services.

Establish an ISAKMP policy and view the available options.

To allow IKE Phase 1 negotiation, you must create an ISAKMP policy and configure a peer association involving that ISAKMP policy. An ISAKMP policy defines the authentication and encryption algorithms, and the hash function used to send control traffic between the two VPN endpoints. When an ISAKMP security association has been accepted by the IKE peers, IKE Phase 1 has been completed. IKE Phase 2 parameters will be configured later.

Issue the crypto isakmp policy number global configuration mode command on R1 for policy 10.

R3(config)# crypto isakmp policy 10

View the various IKE parameters available using Cisco IOS help by typing a question mark (?).

R3(config-isakmp)# ?

ISAKMP commands:

authentication  Set authentication method for protection suite

default         Set a command to its defaults

encryption      Set encryption algorithm for protection suite

exit            Exit from ISAKMP protection suite configuration mode

group           Set the Diffie-Hellman group

hash            Set hash algorithm for protection suite

lifetime        Set lifetime for ISAKMP security association

no              Negate a command or set its defaults

Configure ISAKMP policy parameters on R3.

The encryption algorithm determines how confidential the control channel between the endpoints is. The hash algorithm controls data integrity, which ensures that the data received from a peer has not been tampered with in transit. The authentication type ensures that the packet was sent and signed by the remote peer. The Diffie-Hellman group is used to create a secret key shared by the peers that has not been sent across the network.

Configure an ISAKMP policy with a priority of 10. Use pre-shared key as the authentication type,.3des for the encryption algorithm, sha as the hash algorithm, and the Diffie-Hellman group 2 key exchange.

Note: Older versions of Cisco IOS do not support AES 256 encryption and SHA as a hash algorithm. Substitute whatever encryption and hashing algorithm your router supports. Ensure that the same changes are made on R3 in order to be in sync.

R3(config)# crypto isakmp policy 10

R3(config-isakmp)# authentication pre-share

R3(config-isakmp)# encryption 3des

R3(config-isakmp)# hash sha

R3(config-isakmp)# group 2

R3(config-isakmp)# end

Verify the IKE policy with the show crypto isakmp policy command.

R3# show crypto isakmp policy

Global IKE policy

Protection suite of priority 10

encryption algorithm:   Three key triple DES

hash algorithm:         Secure Hash Standard

authentication method:  Pre-Shared Key

Diffie-Hellman group:   #2 (1024 bit)

lifetime:               86400 seconds, no volume limit

Configure pre-shared keys.

Because pre-shared keys are used as the authentication method in the IKE policy, a key must be configured on each router that points to the other VPN endpoint. These keys must match for authentication to be successful. The crypto isakmp key key-string address ip-address global configuration command is used to enter a pre-shared key and identify the IP address of the remote VPN peer.

Which IP address should you use to configure the IKE peer, given the topology diagram and IP addressing table?

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

The IP address should be the ASA outside IP address of 209.165.200.226.

Each IP address that is used to configure the IKE peers is also referred to as the IP address of the remote VPN endpoint. Configure the pre-shared key of SECRET-KEY on R3. Production networks should use a complex key. This command points to the remote ASA outside IP address.

R3(config)# crypto isakmp key SECRET-KEY address 209.165.200.226

Configure the IPsec transform set and lifetime.

The IPsec transform set is another crypto configuration parameter that routers negotiate to form a security association. It is configured using the crypto ipsec transform-set tag global configuration command. Configure the transform set with the tag ESP-TUNNEL. Use ? to see which parameters are available.

R3(config)# crypto ipsec transform-set ESP-TUNNEL ?

ah-md5-hmac      AH-HMAC-MD5 transform

ah-sha-hmac      AH-HMAC-SHA transform

ah-sha256-hmac   AH-HMAC-SHA256 transform on R3

ah-sha384-hmac   AH-HMAC-SHA384 transform

ah-sha512-hmac   AH-HMAC-SHA512 transform

comp-lzs         IP Compression using the LZS compression algorithm

esp-3des         ESP transform using 3DES(EDE) cipher (168 bits)

esp-aes          ESP transform using AES cipher

esp-des          ESP transform using DES cipher (56 bits)

esp-gcm          ESP transform using GCM cipher

esp-gmac         ESP transform using GMAC cipher

esp-md5-hmac     ESP transform using HMAC-MD5 auth

esp-null         ESP transform w/o cipher

esp-seal         ESP transform using SEAL cipher (160 bits)

esp-sha-hmac     ESP transform using HMAC-SHA auth

esp-sha256-hmac  ESP transform using HMAC-SHA256 auth

esp-sha384-hmac  ESP transform using HMAC-SHA384 auth

esp-sha512-hmac  ESP transform using HMAC-SHA512 auth

In our Site-to-site VPN with the ASA, we will use the two highlitghed parameters. Complete the command by entering the two highlighted parameters.

R3(config)# crypto ipsec transform-set ESP-TUNNEL esp-3des esp-sha-hmac

What is the function of the IPsec transform set?

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

The IPsec transform set specifies the cryptographic algorithms and functions (transforms) that a router employs on the data packets sent through the IPsec tunnel. These algorithms include the encryption, encapsulation, authentication, and data integrity services that IPsec can apply.

Define interesting traffic.

To make use of the IPsec encryption with the VPN, it is necessary to define extended access lists to tell the router which traffic to encrypt. A packet that is permitted by an access list used for defining IPsec traffic is encrypted if the IPsec session is configured correctly. A packet that is denied by one of these access lists is not dropped. The packet is sent unencrypted. Also, like any other access list, there is an implicit deny at the end, which means the default action is to not encrypt traffic. If there is no IPsec security association correctly configured, no traffic is encrypted and traffic is forwarded unencrypted.

In this scenario, from the perspective of R3, the traffic you want to encrypt is traffic going from R3’s Ethernet LAN to the ASA inside LAN or vice versa from the perspective of the ASA.

Configure the IPsec VPN interesting traffic ACL on R3.

R3(config)# ip access-list extended VPN-ACL

R3(config-ext-nacl)# remark Link to the CCNAS-ASA

R3(config-ext-nacl)# permit ip 172.16.3.0 0.0.0.255 192.168.1.0 0.0.0.255

R3(config-ext-nacl)# exit

Does IPsec evaluate whether the access lists are mirrored as a requirement to negotiate its security association?

____________________________________________________________________________________

____________________________________________________________________________________

____________________________________________________________________________________

Yes. IPsec does evaluate whether access lists are mirrored. IPsec does not form a security association if the peers do not have mirrored access lists to select interesting traffic.

Create and apply a crypto map.

A crypto map associates traffic that matches an access list to a peer and various IKE and IPsec settings. After the crypto map is created, it can be applied to one or more interfaces. The interfaces that it is applied to should be the interfaces facing the IPsec peer.

To create a crypto map, use the crypto map name sequence-num type global configuration command to enter crypto map configuration mode for that sequence number. Multiple crypto map statements can belong to the same crypto map and are evaluated in ascending numerical order.

Create the crypto map on R3, name it S2S-MAP, and use 10 as the sequence number. Use a type of ipsec-isakmp, which means IKE is used to establish IPsec security associations. A message displays after the command is issued.

R3(config)# crypto map S2S-MAP 10 ipsec-isakmp

% NOTE: This new crypto map will remain disabled until a peer

and a valid access list have been configured.

R3(config-crypto-map)#

Use the match address access-list command to specify which access list defines which traffic to encrypt.

R3(config-crypto-map)# match address VPN-ACL

Setting a peer IP or hostname is required. Set it to the ASA remote VPN endpoint interface using the following command.

R3(config-crypto-map)# set peer 209.165.200.226

Use the set transform-set tag command to hard code the transform set to be used with this peer.

R3(config-crypto-map)# set transform-set ESP-TUNNEL

R3(config-crypto-map)# exit

Apply the crypto map to interfaces.

Note: The SAs are not established until the crypto map has been activated by interesting traffic. The router generates a notification that crypto is now on.

Apply the crypto maps to the R3 Serial1/0 interface.

R3(config)# interface Serial1/0

R3(config-if)# crypto map S2S-MAP

R3(config-if)# end

R3#

*Feb 29 06:23:03.863: %CRYPTO-6-ISAKMP_ON_OFF: ISAKMP is ON

- Configure the ASA as a Site-to-Site IPsec VPN Endpoint Using ASDM

In Part 4 of this lab, you will configure the ASA as an IPsec VPN tunnel endpoint. The tunnel between the ASA and R3 passes through R1 and R2.

Access ASDM.

On PC-B, start ASDM using the ASDM application or by using a browser and connecting to https://192.168.1.1 and then choosing Run ASDM.

Please refer to the previous lab for more detailed instructions.

Note: If one of the choices is Install Java Web Start, you will need to input https://192.168.1.1/admin/public/startup.jnlp in a browser if you do not want to install the Launcher.

After the ASDM Launcher starts, log in as user admin01 with password admin01pass.

Exit the Startup Wizard if it starts.

Review the ASDM Home screen.

The Home screen displays and shows the current ASA device configuration and traffic flow statistics. Notice the inside, outside, and dmz interfaces that were configured in Part 2 of this lab.

Start the VPN wizard.

On the ASDM main menu, click Wizards > VPN Wizards > Site-to-site VPN Wizard to open the Site-to-site VPN Connection Setup Wizard Introduction window. Review the on-screen text and topology diagram.

Click Next to continue and open the Peer Device Identification window.

Configure peer device identification.

The VPN tunnel will be between R3 S1/0 and the ASA outside interface (G0/0). Therefore, in the Peer IP Address field, enter 10.2.2.1 which is the IP address of the R3 Serial1/0 interface. Leave the default VPN Access Interface set to outside.

Click Next to continue and open the Traffic to protect window.

Specify the traffic to protect.

Click on the Local Network field ellipsis to list possible network objects and double-click on the inside-network/24 (192.168.1.0/24) option.

Click OK to continue.

Click in the Remote Network field and type in 172.16.3.0/24 to identify the R3 LAN as the remote network.

Click Next to continue and open the Security window. A message may display stating that the certificate information is being retrieved.

Note: If the ASA does not respond, you may need to close the window and continue to the next step. If prompted to authenticate, log in again as admin01 with the password admin01pass.

Configure authentication.

In the Security window, we need to enter the same shared secret as R3. Therefore, enter SECRET-KEY. You will not be using a device certificate.

Click Next to continue and open the NAT Exempt window.

Configure miscellaneous settings.

We do now want tunnel traffic to use NAT. Therefore, click the Exempt ASA check box for the inside interface.

Click Next to continue and open the Summary window.

Review the configuration summary and deliver the commands to the ASA.

Verify that the information configured is correct. You can click Back to make changes, or click Cancel and restart the VPN wizard (recommended).

Click Finish to complete the process and deliver the commands to the ASA.

Note: If prompted to authenticate, log in again as admin01 with the password admin01pass.

Verify the ASDM VPN connection profile.

The ASDM Configurations > Site-to-Site VPN > Connection Profiles screen displays the settings you configured. From this window, the VPN configuration can be verified and edited.

Use ASDM monitoring to verify the tunnel.

On the ASDM menu bar, click Monitoring > VPN from the panels at the lower left of the screen.

Click VPN Statistics > Sessions. Notice how there is no active session. This is because the VPN tunnel has not been established.

Test the VPN configuration from PC-B.

To establish the VPN tunnel, interesting traffic must be generated. From PC-B, ping PC-C.

      - This generates interesting traffic. Notice how one or two pings failed before being successful. This is because the tunnel first had to be negotiated and established before the ICMP packets could be successful.

      - The VPN information is now being displayed on the ASDM Monitoring > VPN > VPN Statistics > Sessions page.

Note: You may need to click Refresh before the statistics will display.

Click Encryption Statistics under VPN Statistics. You should see one or more sessions using the 3DES encryption algorithm.

Click Crypto Statistics under VPN Statistics. You should see values for the number of packets encrypted and decrypted, security association (SA) requests, etc.

Reflection

Describe a situation where a site-to-site IPsec VPN would be preferable over other VPN options.

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

_______________________________________________________________________________________

When a large number of hosts exist at a remote office and traffic between the office and a central site needs to be protected. One disadvantage of the site-to-site VPN is that traffic on the remote network (connecting host) is not protected. Only the traffic between the site-to-site tunnel endpoints is protected.

Router Interface Summary Table

ASA 5506-X Config

CCNAS-ASA# show run

: Saved

:

: Hardware:   ASA5506, 4096 MB RAM, CPU Atom C2000 series 1250 MHz, 1 CPU (4 cores)

:

ASA Version 9.10(1)

!

hostname CCNAS-ASA

domain-name ccnasecurity.com

enable password ***** pbkdf2

names

no mac-address auto

!

interface GigabitEthernet0/0

nameif outside

security-level 0

ip address 209.165.200.226 255.255.255.248

!

interface GigabitEthernet0/1

nameif inside

security-level 100

ip address 192.168.1.1 255.255.255.0

!

interface GigabitEthernet0/2

nameif dmz

security-level 70

ip address 192.168.2.1 255.255.255.0

!

!

ftp mode passive

dns server-group DefaultDNS

domain-name ccnasecurity.com

object network inside-net

subnet 192.168.1.0 255.255.255.0

object network dmz-server

host 192.168.2.3

object network NETWORK_OBJ_172.16.3.0_24

subnet 172.16.3.0 255.255.255.0

object network NETWORK_OBJ_192.168.1.0_24

subnet 192.168.1.0 255.255.255.0

access-list OUTSIDE-DMZ extended permit ip any host 192.168.2.3

access-list outside_cryptomap extended permit ip 192.168.1.0 255.255.255.0 172.16.3.0 255.255.255.0

pager lines 24

mtu inside 1500

mtu outside 1500

mtu dmz 1500

icmp unreachable rate-limit 1 burst-size 1

no asdm history enable

arp timeout 14400

no arp permit-nonconnected

arp rate-limit 16384

nat (inside,outside) source static NETWORK_OBJ_192.168.1.0_24 NETWORK_OBJ_192.168.1.0_24 destination static NETWORK_OBJ_172.16.3.0_24 NETWORK_OBJ_172.16.3.0_24 no-proxy-arp route-lookup

!

object network inside-net

nat (inside,outside) dynamic interface

object network dmz-server

nat (dmz,outside) static 209.165.200.227

access-group OUTSIDE-DMZ in interface outside

route outside 0.0.0.0 0.0.0.0 209.165.200.225 1

timeout xlate 3:00:00

timeout pat-xlate 0:00:30

timeout conn 1:00:00 half-closed 0:10:00 udp 0:02:00 sctp 0:02:00 icmp 0:00:02

timeout sunrpc 0:10:00 h323 0:05:00 h225 1:00:00 mgcp 0:05:00 mgcp-pat 0:05:00

timeout sip 0:30:00 sip_media 0:02:00 sip-invite 0:03:00 sip-disconnect 0:02:00

timeout sip-provisional-media 0:02:00 uauth 0:05:00 absolute

timeout tcp-proxy-reassembly 0:01:00

timeout floating-conn 0:00:00

timeout conn-holddown 0:00:15

timeout igp stale-route 0:01:10

user-identity default-domain LOCAL

aaa authentication ssh console LOCAL

aaa authentication http console LOCAL

aaa authentication login-history

http server enable

http 192.168.1.0 255.255.255.0 inside

no snmp-server location

no snmp-server contact

service sw-reset-button

crypto ipsec ikev1 transform-set ESP-AES-128-SHA esp-aes esp-sha-hmac

crypto ipsec ikev1 transform-set ESP-AES-128-MD5 esp-aes esp-md5-hmac

crypto ipsec ikev1 transform-set ESP-AES-192-SHA esp-aes-192 esp-sha-hmac

crypto ipsec ikev1 transform-set ESP-AES-192-MD5 esp-aes-192 esp-md5-hmac

crypto ipsec ikev1 transform-set ESP-AES-256-SHA esp-aes-256 esp-sha-hmac

crypto ipsec ikev1 transform-set ESP-AES-256-MD5 esp-aes-256 esp-md5-hmac

crypto ipsec ikev1 transform-set ESP-AES-128-SHA-TRANS esp-aes esp-sha-hmac

crypto ipsec ikev1 transform-set ESP-AES-128-SHA-TRANS mode transport

crypto ipsec ikev1 transform-set ESP-AES-128-MD5-TRANS esp-aes esp-md5-hmac

crypto ipsec ikev1 transform-set ESP-AES-128-MD5-TRANS mode transport

crypto ipsec ikev1 transform-set ESP-AES-192-SHA-TRANS esp-aes-192 esp-sha-hmac

crypto ipsec ikev1 transform-set ESP-AES-192-SHA-TRANS mode transport

crypto ipsec ikev1 transform-set ESP-AES-192-MD5-TRANS esp-aes-192 esp-md5-hmac

crypto ipsec ikev1 transform-set ESP-AES-192-MD5-TRANS mode transport

crypto ipsec ikev1 transform-set ESP-AES-256-SHA-TRANS esp-aes-256 esp-sha-hmac

crypto ipsec ikev1 transform-set ESP-AES-256-SHA-TRANS mode transport

crypto ipsec ikev1 transform-set ESP-AES-256-MD5-TRANS esp-aes-256 esp-md5-hmac

crypto ipsec ikev1 transform-set ESP-AES-256-MD5-TRANS mode transport

crypto ipsec ikev1 transform-set ESP-3DES-SHA esp-3des esp-sha-hmac

crypto ipsec ikev1 transform-set ESP-3DES-MD5 esp-3des esp-md5-hmac

crypto ipsec ikev1 transform-set ESP-3DES-SHA-TRANS esp-3des esp-sha-hmac

crypto ipsec ikev1 transform-set ESP-3DES-SHA-TRANS mode transport

crypto ipsec ikev1 transform-set ESP-3DES-MD5-TRANS esp-3des esp-md5-hmac

crypto ipsec ikev1 transform-set ESP-3DES-MD5-TRANS mode transport

crypto ipsec ikev1 transform-set ESP-DES-SHA esp-des esp-sha-hmac

crypto ipsec ikev1 transform-set ESP-DES-MD5 esp-des esp-md5-hmac

crypto ipsec ikev1 transform-set ESP-DES-SHA-TRANS esp-des esp-sha-hmac

crypto ipsec ikev1 transform-set ESP-DES-SHA-TRANS mode transport

crypto ipsec ikev1 transform-set ESP-DES-MD5-TRANS esp-des esp-md5-hmac

crypto ipsec ikev1 transform-set ESP-DES-MD5-TRANS mode transport

crypto ipsec ikev2 ipsec-proposal AES256

protocol esp encryption aes-256

protocol esp integrity sha-1 md5

crypto ipsec ikev2 ipsec-proposal AES192

protocol esp encryption aes-192

protocol esp integrity sha-1 md5

crypto ipsec ikev2 ipsec-proposal AES

protocol esp encryption aes

protocol esp integrity sha-1 md5

crypto ipsec ikev2 ipsec-proposal 3DES

protocol esp encryption 3des

protocol esp integrity sha-1 md5

crypto ipsec ikev2 ipsec-proposal DES

protocol esp encryption des

protocol esp integrity sha-1 md5

crypto ipsec security-association pmtu-aging infinite

crypto map outside_map 1 match address outside_cryptomap

crypto map outside_map 1 set peer 10.2.2.1

crypto map outside_map 1 set ikev1 transform-set ESP-AES-128-SHA ESP-AES-128-MD5 ESP-AES-192-SHA ESP-AES-192-MD5 ESP-AES-256-SHA ESP-AES-256-MD5 ESP-3DES-SHA ESP-3DES-MD5 ESP-DES-SHA ESP-DES-MD5

crypto map outside_map 1 set ikev2 ipsec-proposal AES256 AES192 AES 3DES DES

crypto map outside_map interface outside

crypto ca trustpool policy

crypto ikev2 policy 1

encryption aes-256

integrity sha

group 5 2

prf sha

lifetime seconds 86400

crypto ikev2 policy 10

encryption aes-192

integrity sha

group 5 2

prf sha

lifetime seconds 86400

crypto ikev2 policy 20

encryption aes

integrity sha

group 5 2

prf sha

lifetime seconds 86400

crypto ikev2 policy 30

encryption 3des

integrity sha

group 5 2

prf sha

lifetime seconds 86400

crypto ikev2 policy 40

encryption des

integrity sha

group 5 2

prf sha

lifetime seconds 86400

crypto ikev2 enable outside

crypto ikev1 enable outside

crypto ikev1 policy 10

authentication pre-share

encryption aes-256

hash sha

group 2

lifetime 86400

crypto ikev1 policy 20

authentication rsa-sig

encryption aes-256

hash sha

group 2

lifetime 86400

crypto ikev1 policy 40

authentication pre-share

encryption aes-192

hash sha

group 2

lifetime 86400

crypto ikev1 policy 50

authentication rsa-sig

encryption aes-192

hash sha

group 2

lifetime 86400

crypto ikev1 policy 70

authentication pre-share

encryption aes

hash sha

group 2

lifetime 86400

crypto ikev1 policy 80

authentication rsa-sig

encryption aes

hash sha

group 2

lifetime 86400

crypto ikev1 policy 100

authentication pre-share

encryption 3des

hash sha

group 2

lifetime 86400

crypto ikev1 policy 110

authentication rsa-sig

encryption 3des

hash sha

group 2

lifetime 86400

crypto ikev1 policy 130

authentication pre-share

encryption des

hash sha

group 2

lifetime 86400

crypto ikev1 policy 140

authentication rsa-sig

encryption des

hash sha

group 2

lifetime 86400

telnet timeout 5

ssh stricthostkeycheck

ssh 192.168.1.0 255.255.255.0 inside

ssh timeout 10

ssh version 2

ssh key-exchange group dh-group1-sha1

console timeout 0

threat-detection basic-threat

threat-detection statistics access-list

no threat-detection statistics tcp-intercept

group-policy GroupPolicy_10.2.2.1 internal

group-policy GroupPolicy_10.2.2.1 attributes

vpn-tunnel-protocol ikev1 ikev2

dynamic-access-policy-record DfltAccessPolicy

username admin01 password ***** pbkdf2

tunnel-group 10.2.2.1 type ipsec-l2l

tunnel-group 10.2.2.1 general-attributes

default-group-policy GroupPolicy_10.2.2.1

tunnel-group 10.2.2.1 ipsec-attributes

ikev1 pre-shared-key *****

ikev2 remote-authentication pre-shared-key *****

ikev2 local-authentication pre-shared-key *****

!

class-map inspection_default

match default-inspection-traffic

!

!

policy-map type inspect dns preset_dns_map

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

policy-map global_policy

class inspection_default

inspect ftp

inspect h323 h225

inspect h323 ras

inspect ip-options

inspect netbios

inspect rsh

inspect rtsp

inspect skinny

inspect esmtp

inspect sqlnet

inspect sunrpc

inspect tftp

inspect sip

inspect xdmcp

inspect dns preset_dns_map

inspect icmp

policy-map type inspect dns migrated_dns_map_2

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

policy-map type inspect dns migrated_dns_map_1

parameters

message-length maximum client auto

message-length maximum 512

no tcp-inspection

!

service-policy global_policy global

prompt hostname context

no call-home reporting anonymous

call-home

profile CiscoTAC-1

no active

destination address http https://tools.cisco.com/its/service/oddce/services/DDCEService

destination address email callhome@cisco.com

destination transport-method http

subscribe-to-alert-group diagnostic

subscribe-to-alert-group environment

subscribe-to-alert-group inventory periodic monthly

subscribe-to-alert-group configuration periodic monthly

subscribe-to-alert-group telemetry periodic daily

Cryptochecksum:1074c3a8cd1c2fac7b700449f04085d3

: end

Router R1

Building configuration...

Current configuration : 1713 bytes

!

version 15.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R1

!

boot-start-marker

boot-end-marker

!

security passwords min-length 10

enable secret 9 $9$xKxfXtYW7haqI.$gLUxyPBS.RKfwGx.5Nc/yaDEMxvUD9HmhyqDqqwRGWE

!

no aaa new-model

memory-size iomem 15

!

ip domain name ccnasecurity.com

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

cts logging verbose

!

!

username admin01 secret 9 $9$5ShIv6PMmhCYYk$oCVgPgvKu80EKQWTWAj2CWyOkwyl2BHh1PlgxShL/8s

!

redundancy

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

interface Ethernet0/3

ip address 209.165.200.225 255.255.255.248

duplex auto

speed auto

!

interface GigabitEthernet0/1

no ip address

shutdown

duplex auto

speed auto

!

interface Serial1/1

ip address 10.1.1.1 255.255.255.252

clock rate 2000000

!

interface Serial1/0

no ip address

shutdown

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 Serial1/1

!

control-plane

!

line con 0

exec-timeout 5 0

logging synchronous

login local

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

exec-timeout 5 0

logging synchronous

login local

transport input ssh

!

scheduler allocate 20000 1000

!

end

Router R2

R2# show run

Building configuration...

Current configuration : 1678 bytes

!

version 15.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R2

!

boot-start-marker

boot-end-marker

!

security passwords min-length 10

enable secret 9 $9$Nb4BPAMsmT24y.$4bn2kyZCwulndKiaU1453lzF4n3ge95hfoFIKrucvpI

!

no aaa new-model

memory-size iomem 15

!

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

cts logging verbose

!

username admin01 secret 9 $9$6PSI5.sujsrgN.$LFz4TeeqS/1FtxvK23Le8jxUAY9sjeedVmyF/PA9sPo

!

redundancy

!

interface Embedded-Service-Engine0/0

no ip address

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

!

!

interface Serial1/1

ip address 10.1.1.2 255.255.255.252

!

interface Serial1/0

ip address 10.2.2.2 255.255.255.252

clock rate 2000000

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 172.16.3.0 255.255.255.0 Serial1/0

ip route 209.165.200.224 255.255.255.248 Serial1/1

!

control-plane

!

line con 0

exec-timeout 5 0

logging synchronous

login local

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

exec-timeout 5 0

logging synchronous

login local

transport input telnet

!

scheduler allocate 20000 1000

!

end

Router R3

R3# show run

Building configuration...

Current configuration : 2120 bytes

!

version 15.4

service timestamps debug datetime msec

service timestamps log datetime msec

no service password-encryption

!

hostname R3

!

boot-start-marker

boot-end-marker

!

security passwords min-length 10

enable secret 9 $9$bkZXRtRQF6uqE.$MenFawyYEn642ALi3kGNBttfJxBOYk3fScLmuOFO9Tg

!

no aaa new-model

memory-size iomem 15

!

ip domain name ccnasecurity.com

ip cef

no ipv6 cef

!

multilink bundle-name authenticated

!

cts logging verbose

!

vtp domain TSHOOT

vtp mode transparent

username admin01 secret 9 $9$3gQ5keGlFNmNx.$YmVC/lGkYSErJRHSxRZ13xek9HBYHOASaKmQjYeHx36

!

redundancy

!

crypto isakmp policy 10

encr 3des

authentication pre-share

group 2

crypto isakmp key SECRET-KEY address 209.165.200.226

!

crypto ipsec transform-set ESP-TUNNEL esp-3des esp-sha-hmac

mode tunnel

!

crypto map S2S-MAP 10 ipsec-isakmp

set peer 209.165.200.226

set transform-set ESP-TUNNEL

match address VPN-ACL

!

interface Embedded-Service-Engine0/0

no ip address

shutdown

!

!

interface Ethernet0/3

ip address 172.16.3.1 255.255.255.0

duplex auto

speed auto

!

interface Serial1/1

no ip address

shutdown

clock rate 125000

!

interface Serial1/0

ip address 10.2.2.1 255.255.255.252

crypto map S2S-MAP

!

ip forward-protocol nd

!

no ip http server

no ip http secure-server

!

ip route 0.0.0.0 0.0.0.0 Serial1/0

!

ip access-list extended VPN-ACL

permit ip 172.16.3.0 0.0.0.255 192.168.1.0 0.0.0.255

!

control-plane

!

line con 0

exec-timeout 5 0

logging synchronous

login local

line aux 0

line 2

no activation-character

no exec

transport preferred none

transport output pad telnet rlogin lapb-ta mop udptn v120 ssh

stopbits 1

line vty 0 4

exec-timeout 5 0

logging synchronous

login local

transport input ssh

!

scheduler allocate 20000 1000

end

Switches S1, S2 and S3 – Use default configs, except for host name